teacher guide

Electrical circuits 5:

Measuring electricity

# Components

|  | NAME | DESCRIPTION | AUDIENCE |
| --- | --- | --- | --- |
|  | Measuring electricity teachers guide | This guide describes a simple experiment that demonstrates how fuses work. It includes a summary of common electrical safety devices found around the home. | teachers |
|  | Electrical units fact sheet | Charge, current, voltage, power, resistance and energy are described in this series of fact sheets. Worked calculations and focus questions are included. | students |
|  | Electrical safety worksheet | Students answer questions about earthing and safety in household electrical circuits. | students |

Purpose

Students Explain how safety devices and procedures can be put in place to reduce electrical accidents.

# Activity summary

Outcomes

Students:

- perform a procedure to demonstrate the effect of a short circuit in an electrical circuit;

- understand that a fuse is a safety device in a circuit designed to prevent electrical accidents;

- describe safety devices typically used in domestic electrical systems; and

- understand that adequate earthing of appliances prevents serious electrical accidents.

- understand the terms: charge, current, voltage, power, resistance and energy

| ACTIVITY POSSIBLE STRATEGY |  |
| --- | --- |
| Students perform an activity that demonstrates how a fuse protects a circuit. See ‘Fuse experiment’ below. | students work in small groups |
| Students read the fact sheets, Electrical units. Focus question on these fact sheets may be used for class discussion or set as assigments. | individually, small group or class as a whole |
| Students complete the worksheet, Electrical safety, as a follow up to the experiment. | individual |

# Fuse experiment

This is a small group practical activity to demonstrate how a fuse protects a circuit when a large current is permitted to flow (short circuit).

A simple circuit using a 12 V transformer, low current fuse wire, lamp and leads with some wires bare, can be short circuited using a screwdriver or similar conductor. Use protective mats under the fuse wire to prevent damage to bench tops.

Note: Students may be temporarily alarmed by the spark produced when the fuse wire melts.

bare wire	fuse

12 V

bare wire

screwdriver

# Associated SPICE resources

Electrical circuits 5: Measuring electricity may be used with related SPICE resources to address the broader topic of electricity.

| DESCRIPTION | LEARNING PURPOSE |
| --- | --- |
| Electrical circuits (sequence overview) This learning pathway shows how a number of SPICE resources can be combined to teach the topic of electricity. |  |
| Electrical circuits 1: Lightning Students are engaged in the topic of electricity through observing and exploring lightning. | Engage |
| Electrical circuits 2: Static electricity Students explore the effects of charge through a series of laboratory experiments. | Explore 1 |
| Electrical circuits 3: Current electricity Students construct circuits using simple electrical components. | Explore 2 |
| Electrical circuits 4: Circuit rules Students use an interactive learning object to record observations and derive rules for circuits. | Explain 1 |
| Electrical circuits 5: Measuring electricity Models and analogies are introduced to help students understand concepts around properties of electricity and how it is quantified. | Explain 2 |
| Electrical circuits 6: Bioelectricity Four fact sheets and a video provide examples of how electricity is used in living organisms, including humans. | Elaborate |